STANDARD OPERATING PROCEDURE

Title: Trypsin digestion of human serum as background matrix for targeted mass spectrometry analysis. Version #: 1.1; Author: Hui Zhang Laboratory – Johns Hopkins University. Date: 05/08/2014.

# Purpose

The purpose of this document is to describe the method for the enzymatic digestion of serum that will be used as the background matrix for targeted mass spectrometry-based analytical methods.

# Scope

This document describes the detailed procedures for Protein extraction, Trypsin digestion, and Peptide desalting.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures.  The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Equipment

- Vacuum centrifuge

- Temperature-controlled shaking incubator

- Speed-Vac

- NanoDrop spectrophotometer

# Materials

- Human Serum (Sigma-Aldrich; catalog # H4522)

- 1 g 6 cc C18 SPE cartridges (Waters); NOTE: typical binding capacity is 1-5% of the C18 bed weight

# Reagents

- Deionized water

- Urea – Ultra Pure (Thermo Fisher Scientific)

- TCEP (Thermo Fisher Scientific)

- Iodoacetamide (Sigma-Aldrich)

- Optima LC/MS-grade water (Thermo Fisher Scientific)

- Bicinchoninic acid (BCA) protein assay kit (Thermo Fisher Scientific)

- Sequencing-grade modified trypsin (Promega)

- Trifluoroacetic acid (Sigma-Aldrich)

- Ammonium bicarbonate (Sigma-Aldrich)

- Formic acid (Sigma-Aldrich)

- LC/MS-grade water (Thermo Fisher Scientific)

- LC/MS-grade acetonitrile (Thermo Fisher Scientific)

# Solutions

- Urea buffer: 8 M urea in 0.4 M NH4HCO3

- 10 mM TCEP

- 12 mM Iodoacetamide

- 50 % ACN/0.1 % TFA

- 60 % ACN/0.1 % TFA

# Procedure

- Protein extraction and trypsin digestion

  - Dilute serum 1:10 with urea buffer (8 M urea, 0.8 M NH4HCO3, pH 8.0).

  - Measure the protein concentration using the BCA protein assay kit.

  - Reduce disulfide bonds by adding DTT to the sample at a final concentration of 10 mM. Incubate the sample at 37 °C for 1 h.

  - Carbamidomethylate cysteine residues by adding Iodoacetamide at a final concentration of 12 mM. Incubate the sample at RT for 1 h in the dark.

  - Dilute the sample 1:4 with deionized water to reduce the urea concentration to 2 M.

  - Digest the protein with trypsin using an enzyme-to-substrate ratio of 1:50 (wt/wt) for 12 h at 37 °C with mild shaking.

  - Add another aliquot of the same amount of trypsin used in step 6 and incubate overnight at 37 °C with mild shaking.

  - Acidify the sample by the addition of trifluoroacetic acid to a final amount of 10 % (vol/vol) and vortex. Make sure that the pH of the solution is ≤3 using a pH indicator strip before desalting.

- Peptide de-salting

  - Activate the C18 SPE column with 12 mL of 100 % ACN.

  - Condition the C18 SPE column with 9 mL of 50 % ACN/0.1 % TFA.

  - Equilibrate the C18 SPE column with 20 mL of 0.1 % TFA.

  - Load the sample onto the C18 SPE column.

  - Wash the C18 SPE column with 20 mL of 0.1 % TFA.

  - Elute peptides with 6 mL of 60 % ACN/0.1 % TFA and dry in Speed-Vac.

# Referenced Documents

1.  Sun S, Zhou JY, Yang W, Zhang H. Inhibition of protein carbamylation in urea solution using ammonium-containing buffers. Anal Biochem. 2014 Feb 1;446:76-81. doi: 10.1016/j.ab.2013.10.024. Epub 2013 Oct 23. PubMed PMID: 24161613; PubMed Central PMCID: PMC4072244.