# STANDARD OPERATING PROCEDURE

Title: Trypsin digestion of Cell Lysate. Version #: 1; Author: Akhilesh Pandey Lab. Date: 01/15/2016.

# Purpose

The purpose of this document is to describe the enzymatic digestion and subsequent desalting of a cell lysate to prepare proteomic samples that are compatible with mass spectrometry analysis.

# Scope

This procedure may be used to reduce, alkylate, proteolyze, and desalt cell lysate samples.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures.  The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Equipment

- 37°C heat block

# Materials

- Sep-Pak C18 plus light cartridge, 130 mg Sorbent per cartridge, 55-105 µM particle size  (Waters; cat. # WAT023501)

# Reagents

- SigmaUltra Iodoacetamide (Sigma-Aldrich, I1149-5G)

- DL-Dithiothreitol (Sigma-Aldrich; cat. # D9779-10G)

- Trypsin Gold (Promega, V5280)

- Tetraethylammonium bicarbonate (CH3CH2)4N(HCO3) (Sigma-Aldrich; 1268-25G-F)

- Trifluoracetic Acid (Sigma-Aldrich; cat. # T6508-500ML)

- Water: Optima LC/MS-grade (Fisher Scientific; cat. # W6-4)

- Acetonitrile: Optima LC/MS-grade (Fisher Scientific; cat. # A955-4)

- Formic Acid: Pierce Formic Acid, LC-MS grade (Pierce; cat. # 28905)

# Solutions

- 200 mM Iodacetamide (IAM) Stock Solution

  - Suspend 36.8 mg of Iodoacetamide into 1 mL of HPLC water. Mix until fully dissolved.

  - Prepare immediately prior to use. Keep out of light.

- 100 mM Dithiothreitol (DTT) Stock Solution

  - Resuspend 18.4 mg in 1 mL HPLC water. Mix until fully dissolved.

  - Prepare immediately prior to use.

- Trypsin Stock Solution

  - Resuspend 20 µg lyophilized trypsin in 0.2 mL HPLC water (final concentration of enzyme after suspension of 0.1 µg/µL)

- 50 mM Tetraethylammonium bicarbonate (TEABC) (CH3CH2)4N(HCO3)

  - Dilute 1M TEABC stock solution 1:20 (vol/vol) with HPLC water

- 20% Trifluroacetic Acid (TFA)

  - Dilute TFA stock solution 1:5 (vol/vol) with HPLC water

# Procedure

- Thaw cell lysate in 8M Guanidine HCL on ice for processing. Once thawed completely, vortex to mix thoroughly.

- Protein Reduction and Alkylation

  - Add 100 mM DTT stock solution to an aliquot of thawed cell lysate in sufficient volume as to yield to a final concentration of 10mM DTT.

  - Incubate cell lysate and DTT mixture at 60°C for 30 min to reduce protein disulfide bonds. Allow mixture to equilibrate to room temperature.

  - Add 200mM IAM stock solution to lysate mixture in sufficient volume as to yield a final concentration of 20 mM IAM.

  - Incubate mixture at room temperature for 20 min in the dark to alkylate thiols of protein cysteines.

- Protein Trypsinization

  - Add 50 mM TEABC to protein mixture in sufficient volume so as to dilute the initial 8M Guanidine HCL storage buffer concentration (see “SOP: Cell lysate preparation” for solution recipe) to a concentration below 2M. Add additional TEABC as required until the pH of the diluted solution is between 7.0 and 8.0.

  - Add sufficient volume of the trypsin solution to each digest to achieve a 1:20 enzyme:protein substrate ratio.

  - Incubate protein solution at 37°C with mixing for 16 hours to facilitate proteolysis.

- Acidification of Digested Cell Lysate

  - To quench trypsin reaction, dilute peptide digest in 20% TFA so that the pH of the final solution is under 3 and the final concentration of TFA is 1.0%.

  - After acidification, allow precipitate to form by incubating digest on ice for 15 min.

  - Centrifuge acidified peptide solution at 13,200xg (or max speed) for 10 min at 4°C to remove precipitate. Transfer peptide-containing supernatant into new tube.

- Prime Sep-Pak Cartridge

  - Connect a 10 cc reservoir to the short end of the Sep-Pak cartridge.

  - Condition the column with 5.0 mL 100% ACN.

  - Wash column with additional 5.0 mL ACN, 0.1% TFA.

  - To equilibrate column, wash column sequentially with 1 ml, 3 mL, 5 mL, and 6 mL of 0.1% TFA.

- Peptide Purification

  - Load acidified digest prepared in Step 4c onto equilibrated Sep-Pak cartridge.

  - Wash column sequentially with 1ml, 3mL, 5 mL, and 6 mL of 0.1% TFA.

  - Place Sep-Pak column above new 15 mL falcon tube to collect eluent.

  - Add 3.0 mL of 0.1% TFA, 40% ACN to Sep-Pak column to dilute desalted peptides off Sep-Pak column, collecting eluent. Repeat, collecting both eluents in a single tube.

  - Lyopholize peptide solution to dryness to ensure TFA has been removed from the peptide sample.

  - Samples can be stored lyophilized at -80°C until ready for PRM analysis.

- Reconstitute samples prior to LC-PRM

  - Re-constitute dried and desalted peptide digests with 0.1% formic acid to achieve a concentration of 2mg/mL of peptide.

  - Vortex sample to resuspend thoroughly and transfer desired peptide amount to microcentrifuge tube or 96-well plate.

# Referenced Documents

- SOP: Cell lysate preparation