# STANDARD OPERATING PROCEDURE

Title: Trypsin Digestion of Cell Lysate, using automated liquid handler. Version #: 1; Author: Paulovich Lab. Date: 5/1/2012.

# Purpose

The purpose of this document is to describe enzymatic digestion of a cell lysate for protein analysis compatible with mass spectrometry.

# Scope

This procedure may be used to reduce, alkylate, and proteolyze samples.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures.  The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Equipment

- EpMotion Automated Pipetting System (Eppendorf).

# Materials

- Trypsin Gold, (Promega, V5280)

- Urea, (Sigma, U0631)

- 0.5M TCEP, (Pierce, 77720)

- Iodoacetamide (IAM), (Sigma, A3221-10VL)

- 1M Tris (pH8.0), (Sigma, T2694)

- EDTA (Sigma, E7889)

- EGTA (Sigma, E0396)

- Water, HPLC grade (Fisher, W5-1)

- Acetonitrile, HPLC grade (Fisher, A998-1)

- Formic Acid (EDM, 11670-1)

- Sigma Phosphatase Inhibitor Cocktail 1 (Sigma, P2850)

- Sigma Phosphatase Inhibitor Cocktail 2 (Sigma, P5726)

- Deep-well 96-well plate (Eppendorf, 0030 502.248)

- Oasis HLB 96-well plate 30μm (5mg) (Waters, 186000309)

- Square well deep-well V-bottom 96-well plate (Thermo, 95040450)

# Solutions

- Lysis Buffer. Must be made fresh daily:

  - 4 Parts 7.5M Urea (see below).

  - 1 Part 5x Lysis Buffer Stock Solution (see below).

  - Add 1% Sigma phosphatase cocktail 1.

  - Add 1% Sigma phosphatase cocktail 2.

  - Mix well.

- Lysis Buffer with TCEP.  Must be made fresh daily:

  - Add 10% 0.5M TCEP to Lysis Buffer.

  - Mix well.

- 5x Lysis Buffer Stock Solution. May be made in advance and stored at room temp.

  - 12.5mL 1M Tris (pH8.0).

  - 1.0mL 0.5M EDTA.

  - 1.0mL 0.5M EGTA.

  - Add HPLC water to 100mL.

  - Sterilize with 0.22μm filter.

- 7.5 M Urea. Must be made fresh daily:

  - Add 4.50g Urea to a 15mL Falcon tube.

  - Add 6mL HPLC water and mix until Urea is in solution.

  - Add HPLC water to a final volume of 10mL.

- 0.2M Tris, pH8.0:

  - 4 parts HPLC water.

  - 1 part 1M Tris, pH8.0.

- 0.5M Iodoacetamide (IAM) Stock Solution.  Prepare immediately before use and keep out of light:

  - To one 56mg vial of iodoacetamide, add 605L of 0.2M Tris pH 8. (see above).

  - Mix until dissolved.

- Trypsin Gold (Promega):

  - 100μg resuspended in 1mL of 0.2M Tris pH 8. (see above).

- Quench:

  - 4 parts HPLC water.

  - 1 part formic acid.

# Procedure

Preparation of Samples

- Dilute the cell lysate to 2mg/mL with lysis buffer.

- Move 50uL of cell lysate to a deep well plate.

- Add 33uL of lysis buffer containing 50mM TCEP.

- Incubate with mixing at 600rpm for 30 min at 37 °C.

- Add 7μL of 0.5 M IAM, to yield an IAM concentration

- of ~40 mM.

- Alkylate at room temperature for 30 min in the dark.

- Add 740uL of 100mM Tris, pH 8.0 to each tube to decrease the urea concentration to 0.6M.

- Add 20uL trypsin to each digest to achieve a 1:50 enzyme-to-substrate ratio for the 50ug total protein present.

- Incubate with mixing at 600rpm for 2 h at 37 °C.

- Add 10uL trypsin to each digest to achieve a 1:100 enzyme-to-substrate ratio.

- Incubate with mixing at 600rpm for 16 h at 37 °C.

- Add 45μL of 20% FA to each digest to quench the digestion for a final acid concentration of 1%.

Heavy SIS spike preparation

- The stock heavy SIS mix is at 100nM.

- 1mL of stock is thawed and serially diluted 3x by mixing 330uL into 660uL 0.1% FA in 3% ACN.

- 100uL of each concentration point (except for the first dilution, or 33.3nM) are mixed into the three replicates of the samples.

- The top point should represent 10pmol SIS in 50ug lysate, or 200fmol/ug.

Light SIS spike preparation

- The stock light SIS mix is at 50nM.

- 5uL of the stock is mixed into each response curve sample as well as one of the two blank samples.

- This should represent 250fmol SIS in 50ug lysate, or 5fmol/ug.

Desalting Samples Offline by Positive Pressure

- Set the system pressure to 80psi.

- Condition cartridge with 3 x 400uL of 0.1% formic acid in 80% ACN at 12psi.

- Equilibrate cartridge with 4 x 400uL of 0.1% formic acid in 100% water at 12psi.

- Add sample to cartridge at 12psi.

- Wash cartridge with 4 x 400uL of 0.1% formic acid in 100% water at 6psi.

- Elute peptides with 3 x 400uL 0.1% formic acid in 80% acetonitrile at 3psi into a deep well plate.

- Freeze eluates on dry ice or at -80°C for approximately 1 hour.

- Lyophilize samples overnight to dryness.

- Samples can be stored lyophilized at -80°C until ready for MRM analysis.

Reconstituting Samples (To be performed just prior to executing LC-MRM)

- Reconstitute dried and desalted digests with 100uL of 3% acetonitrile, 0.1% formic acid to each sample to achieve 1μg/μL digest solution for the cell lysates.

- Vortex sample, spin down, and transfer ~95μL to an autosampler vial.