# STANDARD OPERATING PROCEDURE

Title: Response Curve. Version #: 3; Author: PNNL Lab. Date: 06/01/2016.

# Purpose

The purpose of this document is to describe the characterization of a set of assays by response curve.

# Scope

This procedure covers overall preparation and running of samples for generating the response curve.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures.  The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Equipment

Milli-Q Integral Water Purification System (EMD Millipore, Darmstadt, Germany, Catalog # ZRXQ010TO)

Centrifuge 5415D (Eppendorf AG, Hamburg, Germany)

Vortex-genie 2 Lab Mixer (Scientific Industries Inc., Bohemia, NY, Catalog # G-560)

Thermomixer R (Eppendorf AG, Hamburg, Germany, Catalog # 5355)

# Materials

Flat-Top Graduated Microcentrifuge Tubes (Fisher Scientific, Asheville, NC, Catalog # 02-681-268 & 02-681-331 & 02-681-332)

Waters Total Recovery Glass Vial (Waters, Milford, MA, Catalog # 186002805)

Waters Vial Cap (Waters, Milford, MA, Catalog # 186000305)

# Solutions

Mobile phase A: 0.1% FA in H2O

Mobile phase B: 0.1% FA in ACN

Washing Buffer 1 (WB1):0.3M Phthalic acid in 80% ACN/5%TFA

Washing Buffer 2 (WB2): 80% ACN/5%TFA

Elution Buffer (EB): 5% NH4OH in 50% ACN, pH ≥10.

# Reagents

Acetonitrile, Optima LC/MS Grade (Fisher Scientific, Asheville, NC, Catalog # A955-4)

Acetonitrile/Formic Acid 0.1% Mobile Phase (EMD Millipore, Darmstadt, Germany, Catalog # FX0437P-1)

Formic Acid, LC-MS Grade (Thermo Fisher Scientific, Waltham, MA, Catalog # 28905) Custom Peptide (New England Peptide Inc., Gardner, MA)

Phthalic Acid, >=99.5% (Sigma Aldrich, St. Louis, MO, Catalog # 402915)

Trifluoroacetic Acid, ReagentPlus 99% (Sigma Aldrich, St. Louis, MO, Catalog # T6508)

Ammonium formate,  for HPLC >=99.0% (Sigma Aldrich, St. Louis, MO, Catalog # 540-69-2)

Ammonium Hydroxide Solution, 28.0-30.0% (Sigma Aldrich, St. Louis, MO, Catalog # 221228)

Peptide Standards:

Both pure heavy stable isotope-labeled peptides and sequence matched pure light versions were synthesized. Heavy peptides incorporated a fully atom labeled 13C and 15N isotope at the C-terminal lysine (K) or arginine (R) position of each (tryptic) peptide, resulting in a mass shift of +8 or +10 Da, respectively. Those pure peptides were purified to >95% purity by HPLC from the vendor. They were quantified by amino acid analysis and aliquots were stored in 5% acetonitrile/0.1% formic acid at -80°C until use. Pure light peptides are spiked in as internal standards (IS). The stock of light internal standard was stored in -80 ºC freezer. Different heavy peptides were spiked in at different concentration level depending on the response of peptides.

Matrix:

A background matrix consisting of ovarian cancer tumor cell digest was freshly prepared. Cell sample was processed as described in SOP TP-1 (Cell sample Preparation). Digestion was performed according to SOP TD-1 (Trypsin Digestion of cell lysis sample). The cell digest was aliquoted and stored in -80 for the response experiment.

# Procedure:

Preparation of Phosphopeptide Samples for LC-MRM by TiO2 Enrichment

- The following is designed to create 9-13 points of varying concentrations of analyte (pure heavy labelled peptide) depending on the peptide response and 1 blank.

- The pure heavy peptide stock is serially diluted with cell digest matrix (0.1 µg/µl).

- 4 µl of each concentration point of heavy stock is added to 34 µL of the digested cell lysis matrix. (By doing this, heavy peptide standard only account for less than 10% of final total volume).

- Each 200 µg cell sample digestions were spiked in 60 fmol light IS peptides and different amount heavy peptides, and speed-vacuum to dry.

- Pack TiO2 beads column (peptide: beads=1:12) by adding beads solution (10ug/µl beads in ACN solution) and centrifuge 3000g for 3 min.

- Resuspend sample with 150 uL WB1 and Centrifuge at 3000xg for 10 min at 4C.

- Load the sample supernatants to column by centrifuging at 500xg for 15min at 4C, save the Flow-through for next loading.

- Wash column with 100uL WB1 by centrifuging at 1000xg for 15min at 4.

- Re-load the first flow-through sample to column column by centrifuging at 500xg for 15min at 4C.

- Wash column with 150 µl WB1 twice by centrifuging at 1000xg for 15min at 4.

- Wash column with 150 µl WB2 twice by centrifuging at 1000xg for 15min at 4.

- Elute column with 100 µl EB twice by centrifuging at 1000xg for 15min at 4.

- Elute solution was dried out by speed-vacuum.

- Each samples were dissolved by 25 ul 0.1% FA solution. Shake the vial on thermomixer with 800 rpm, 4 ºC for 10 min. All samples are prepared in Waters glass vial.

- Put all samples into LC autosampler and get ready for LC-MRM detection (See SOP LC-1 for Liquid Chromatography and SOP PM-1 for Peptide MRM on TSQ Vantage).

- 4 µl of sample is used for each run with the run order of blank, low concentration to high concentration as a batch, and acquire the data in three replicates at each concentration point.

# Referenced Documents

SOP TD-1 for Trypsin Digestion of cell sample.pdf

SOP TP-1 for cell Sample Preparation.pdf

SOP LC-1 for Liquid Chromatography.pdf
SOP PM-1 for Peptide MRM on TSQ Vantage.pdf