# STANDARD OPERATING PROCEDURE

Title: Trypsin Digestion of Cell Lysates. Version #: 2018; Author: Paulovich Lab.

# Purpose

The purpose of this document is to describe the trypsin digestion of cell lysates for samples compatible with mass spectrometry.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures. The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Materials and Reagents

- Lys-C (Endoproteinase Lys-C, Wako No. 129-02541)

- Trypsin, sequencing grade, (Promega, V5113)

- Urea, (Sigma, U0631)

- 0.5 M TCEP, (Pierce, 77720)

- Iodoacetamide (IAM), (Sigma, A3221-10VL) (note these are single-use vials. Alternative to weigh out amounts from another source.)

- 1 M Tris (pH8.0), (Sigma, T2694)

- EDTA (Sigma, E7889)

- EGTA (Sigma, E0396)

- Water, HPLC grade (Fisher, W5-1)

- Acetonitrile, HPLC grade (Fisher, A998-1)

- Formic Acid (EDM, 11670-1)

- Sigma Phosphatase Inhibitor Cocktail 2 (Sigma, P5726)

- Sigma Phosphatase Inhibitor Cocktail 3 (Sigma, P0044)

- 1.2 mL polypropylene 96 well deep well plates (ThermoFisher Scientific, 95040450); used for digestion

- XPierce pre-cut pierce-able films (Genesee Scientific, 12-140), or equivalent ; for digestions

- Thermomixer, or equivalent

# Special Reagents

- Light and stable isotope-labeled heavy synthetic peptides for MRM analyses were purchased at >95% pure as measured by HPLC (“pure”) or ~75% purity (“crude”). Cysteine residues of the tryptic light and heavy peptides were carbamidomethylated, and the heavy peptides’ C-terminal lysine (K) or arginine (R) residues were uniformly 13C and 15N labeled, yielding a mass increase of 8 Da (for K) or 10 Da (for R). For peptides at the C-terminus of proteins, other amino acids (such as valine) can be isotopically labeled. Individual peptide stock aliquots were stored in 30% acetonitrile-0.1% formic acid at 100 pmol/L at -80 °C.

- Rabbit-anti-human and mouse-anti-human monoclonal antibodies (mAbs) were custom-generated to the tryptic and proteotypic peptide targets and covalently coupled to magnetic Protein G beads.

# Solutions

- 5 × Lysis Buffer Stock Solution. May be made in advance and stored at room temp.

  - 12.5 mL 1M Tris (pH 8.0).

  - 1.0 mL 0.5 M EDTA.

  - 1.0 mL 0.5 M EGTA.

  - Add HPLC water to 100 mL.

  - Filter with 0.22 μm filter.

- 7.5 M Urea. Must be made fresh daily:

  - Add 4.5 g Urea to a 15 mL Falcon tube.

  - Add 6 mL HPLC water and mix until Urea is in solution.

  - Add HPLC water to a final volume of 10 mL.

- Lysis Buffer. Must be made fresh daily:

  - 4 Parts 7.5 M Urea (see above).

  - 1 Part 5 × Lysis Buffer Stock Solution (see above).

  - Add 1% Sigma phosphatase cocktail 2.

  - Add 1% Sigma phosphatase cocktail 3.

  - Mix well.

- 0.2 M Tris, pH 8.0:

  - 4 parts HPLC water.

  - 1 part 1M Tris, pH 8.0.

- 30 mM TCEP Solution. Must be made fresh daily:

  - Dilute 0.5 M TCEP solution to 30 mM using lysis buffer

- 0.5 M Iodoacetamide (IAM) Stock Solution.  Prepare immediately before use and keep out of light:

  - To one 56 mg vial of iodoacetamide, add 605 μL of 0.2 M Tris buffer.

  - Mix until dissolved. Visually inspect vial to confirm fully solvated.

- Lys-C (Assuming 8 mg in 10 AU):

  - Prepare 50 mM Acetic Acid by mixing:

  - 17.8 mL HPLC water.

  - 7.2 mL 1% solution of Acetic Acid (HOAc, Sigma cat# 242853, stock conc. 17.4 M)

  - Dissolve 10 AU Lys-C in 6 mL 50 mM HOAc.

  - Transfer dissolved enzyme to a 50 mL tube.

  - Add 10.0 mL 50 mM HOAc for a final volume of 16 mL and an estimated conc. of 500 μg/mL.

  - Aliquot 0.5 mL/1.7 mL microfuge tube (~250 ug / tube)

  - Store aliquots at -80 °C.

  - Dilute to 0.2 mg/mL in 0.2 M Tris pH 8.

- Trypsin:

  - 100 μg resuspended in 500 μL of 0.2 M Tris pH 8.

- Quench:

  - 4 parts HPLC water.

  - 1 part formic acid.

# Procedure

- Transfer 20-100 μg of tumor tissue lysate to a 1.2 mL deep well plate.

- Place X-Pierce film on sample plate.

- Add 33 μL of 30 mM TCEP in lysis buffer.

- Incubate with mixing at 600 rpm on a Thermomixer for 30 min at 37 °C.

- Add 36 μL of 0.5 M IAM in Lysis buffer.

- Alkylate at room temperature for 30 min in the dark.

- Add 740 μL of 0.2 M Tris, pH 8.0 to each tube to decrease the urea concentration to ~0.6 M.

- Add 20 μL LysC to each digest to achieve a 1:50 enzyme-to-substrate ratio.

- Incubate with mixing at 600 rpm for 2 h at 37 °C.

- Add 10 μL trypsin to each digest to achieve a 1:100 enzyme-to-substrate ratio.

- Incubate with mixing at 600 rpm for 16 h at 37 °C.

- Add 45 μL of 20% FA to each digest to quench the digestion for a final acid concentration of ~1%.

- Add 10 μL of heavy SIS mix working solution to each sample.

- Cleanup (i.e. desalt) the sample using Oasis cartridge.

- Lyophilize and store at -80C prior to immunoaffinity enrichment.

# Referenced Documents

NA