# STANDARD OPERATING PROCEDURE

Title: Peptide immunoaffinity enrichment for the characterization of immuno-MRM assays using response curves. Version #: 2018; Author: Paulovich Lab.

# Purpose

The purpose of this document is to describe generation of response curves for characterization of linearity and lower limits of quantification (LLOQ).

# Scope

This procedure may be used to generate immunoaffinity-enriched peptide response curve samples to be analyzed by MRM. It includes a description of generating and analyzing the response curve samples.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures. The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Equipment

- KingFisher 96 magnetic bead handling platform (5400500, Thermo Scientific), equipped with a PCR magnet head

- LabQuake tube rotator (415110, Barnstead)

- Dynabeads® MPC®-S (Magnetic Particle Concentrator, A13346, Life Technologies) or DynaMagTM-15 Magnet (12301D, Life Technologies)

- DynaMagTM-96 Side Skirted Magnet (12027, Life Technologies)

- Centrifuge for use with 96 well plates (5810R, Eppendorf)

# Materials

- 1.7 mL microcentrifuge tubes (24-282, Genesee Scientific)

- 15 mL conical centrifuge tubes (352196, Corning Life Sciences)

- MColorpHast® pH Test Strips, 5-10 (1.09533.0001, EMD Millipore)

- KingFisher 96 KF microplate (200 L) (97002540, Thermo Scientific)

- KingFisher 96 tip comb for PCR magnets (97002514, Thermo Scientific)

- Hard-Shell® Low-Profile Thin-Wall 96-Well Skirted PCR Plates (HSP-9601, BioRad)

- Chemically resistant sealing foil (T-3025-8B, BioExpress)

- ThermalSeal® PCR Sealing Films (12-168, Genesee Scientific)

- 1 M Trizma hydrochloride buffer, pH 8.0 (Tris, T2694, Sigma)

- 10X Phosphate buffered saline (PBS) (BP39920, Fisher Scientific)

- CHAPS detergent (28300, Thermo Scientific)

- Water, Optima® LC/MS, suitable for UHPLC-UV (W6-4, Fisher Scientific)

- Acetic acid, ACS reagent, ≥99.7% (242853, Sigma)

- Formic acid (FA) (#11670-1, Millipore)

- Acetonitrile, Optima® LC/MS, suitable for UHPLC-UV (ACN) (A955-4, Fisher Scientific)

# Reagents

- Light and stable isotope-labeled heavy synthetic peptides for MRM analyses were purchased at >95% pure as measured by HPLC (“pure”) or ~75% purity (“crude”). Cysteine residues of the tryptic light and heavy peptides were carbamidomethylated, and the heavy peptides’ C-terminal lysine (K) or arginine (R) residues were uniformly 13C and 15N labeled, yielding a mass increase of 8 Da (for K) or 10 Da (for R). For peptides at the C-terminus of proteins, other amino acids (such as valine) can be isotopically labeled. Amino acid analysis (AAA) was performed to determine peptide concentrations. Individual peptide stock aliquots were stored in 0.1% FA in 30% ACN at 100 pmol/L at -80 °C.

- Rabbit-anti-human and mouse-anti-human monoclonal antibodies (mAbs) were custom-generated to the tryptic and proteotypic peptide targets and covalently coupled to magnetic Protein G beads.

- Background matrix (e.g. tissue lysate or plasma, as appropriate for sample set)

# Solutions

- Prepare a 0.1% FA (v/v), 3% ACN solution in Optima® water.

- Prepare a 30 mg/mL trypsin digested yeast lysate in 0.1% FA, 3% ACN by diluting a 1 mg/mL solution of digested yeast lysate into 0.1% FA, 3% ACN solution.

# Procedure

The following describes the experiments for a response curve that has varying concentrations of the heavy (analyte) peptides and a constant concentration of the light (internal standard) peptides in a human tissue lysate matrix, including a 0 fmol (blank) sample that does not contain heavy peptides. The light peptides were not chosen as analytes since the endogenous detection of some of the peptides in background matrix would not have made it possible to determine the limits of detection and quantification.

Preparation of the response curve samples

This procedure describes heavy peptides that were tested in these particular response curves at 1000, 200, 40, 8, 3.2, 1.28, 0.512, 0.2048, and 0 fmol per 100 g of lysate digest. These levels were generated using serial dilutions, with the dilutions from 1000 to 8 fmol/sample being covered by 1:5 serial dilutions, and from 8 to 0.2048 fmol/sample by 1:2.5 serial dilutions (smaller intervals were chosen for the lower levels since this would allow more robust characterizations of the limits of quantification). Where available, the synthetic light peptides were spiked at a constant level of 80 fmol per sample.

- Spike light SIS peptide mix into curve concentration points:

1.1. 	Dilute stock light SIS peptide mix to 8 nM in 30 mg/mL trypsin digested yeast lysate in 0.1% FA, 3% ACN.

1.2. 	Spike 10 mL of diluted light SIS into each concentration point, including the 0 fmol point.

- Spike heavy SIS peptide mix into curve concentration points:

2.1.	Dilute stock heavy SIS peptide mix to 100 nM in 30 mg/mL trypsin digested yeast lysate in 0.1% FA, 3% ACN.

2.2.	Serially dilute the 100 nM heavy SIS mix using 30 mg/mL trypsin digested yeast lysate in 0.1% FA, 3% ACN to the following concentration points: 20, 4, 0.8, 0.32, 0.128, 0.0512 and 0.02048 nM.

2.3.	Spike 10 mL of serially diluted heavy SIS into individual concentration points.

2.4.	Spike 10 mL of 30 mg/mL trypsin digested yeast lysate in 0.1% FA, 3% ACN into the 0 fmol.

Perform immunoaffinity enrichment (capture) of the peptides from the digest samples according to the enrichment SOP and analyze by LC-MRM.

Sample run order on the LC-MRM-MS system: The response curve samples are run in increasing order of heavy peptide level, with one injection per capture replicate. Before that, a system suitability standard peptide mix is run to ascertain that the system is behaving as expected (see also Abbatiello, S. E., et al.1). An injection of the synthetic light peptide standards in buffer is then performed to determine the approximate retention times of the peptides, with which a scheduled LC-MRM method is generated using the Skyline software2. This is followed by the samples, with a buffer blank run (injecting 8 L of 3% acetonitrile-5% acetic acid) in between each sample run. (Due to some peptides showing considerable carry-over (e.g. >1%) from run to run, it is not practical to randomize the run order with respect to heavy peptide spike level.)

# Referenced Documents

1.	Abbatiello, S. E., Mani, D. R., Schilling, B., Maclean, B., Zimmerman, L. J., Feng, X., Cusack, M. P., Sedransk, N., Hall, S. C., Addona, T., Allen, S., Dodder, N. G., Ghosh, M., Held, J. M., Hedrick, V., Inerowicz, H. D., Jackson, A., Keshishian, H., Kim, J. W., Lyssand, J. S., Riley, C. P., Rudnick, P., Sadowski, P., Shaddox, K., Smith, D., Tomazela, D., Wahlander, A., Waldemarson, S., Whitwell, C. A., You, J., Zhang, S., Kinsinger, C. R., Mesri, M., Rodriguez, H., Borchers, C. H., Buck, C., Fisher, S. J., Gibson, B. W., Liebler, D., MacCoss, M., Neubert, T. A., Paulovich, A., Regnier, F., Skates, S. J., Tempst, P., Wang, M., and Carr, S. A. (2013) Design, implementation and multisite evaluation of a system suitability protocol for the quantitative assessment of instrument performance in liquid chromatography-multiple reaction monitoring-MS (LC-MRM-MS). Mol Cell Proteomics 12, 2623-2639.

2.	MacLean, B., Tomazela, D. M., Shulman, N., Chambers, M., Finney, G. L., Frewen, B., Kern, R., Tabb, D. L., Liebler, D. C., and MacCoss, M. J. (2010) Skyline: an open source document editor for creating and analyzing targeted proteomics experiments. Bioinformatics 26, 966-968.