# STANDARD OPERATING PROCEDURE

Title: Trypsin digestion of serum and enrichment of N-glycopeptides using an automated liquid handler. Version #: 1; Author: Hui Zhang Lab. Date: 05/08/2014.

# Purpose

The purpose of this document is to describe the enzymatic digestion of serum and the subsequent solid phase enrichment of N-glycopeptides using an automated liquid handler to prepare samples that are compatible with mass spectrometry analysis.

# Scope

This procedure is used to reduce, alkylate, proteolyze and enrich N-linked glycopeptides from serum.

# Responsibilities

It is the responsibility of person(s) performing this procedure to be familiar with laboratory safety procedures.  The interpretation of results must be done by a person trained in the procedure and familiar with such interpretation.

# Equipment

- Versette liquid handling system (Thermo Fisher Scientific)

- Vacuum centrifuge

# Materials

- Disposable automation research pipette tips (Thermo Fisher Scientific)

- 96-well plate

- Affi-gel hydrazide gel (Bio-Rad)

- Frits (2 mm diameter, 1 mm thick, 15 – 45 µm pores) (POREX)

- 7k MWCO Zeba spin desalting columns (Thermo Fisher Scientific)

- Human Serum (Sigma-Aldrich)

# Reagents

- Sodium periodate (Bio-Rad)

- Urea (Thermo Fisher Scientific)

- Tris(2-carboxyethyl)phosphine (Thermo Fisher Scientific)

- Iodoacetamide (Sigma-Aldrich)

- Sequencing-grade trypsin (Promega)

- PNGase F (New England Biolabs)

- Sodium acetate (Sigma-Aldrich)

- Sodium chloride (Sigma-Aldrich)

- Aniline (Sigma-Aldrich)

- Ammonium bicarbonate (NH4HCO3) (Sigma-Aldrich)

- Acetonitrile (Fisher Scientific)

- Formic acid (Fisher Scientific)

- Water (Fisher Scientific)

- Trifluoroacetic acid (Thermo Pierce Scientific)

# Solutions

- Coupling buffer: 500 mM sodium acetate, 0.3 mM sodium chloride, pH 5

- Oxidation buffer: 15 mM sodium periodate

- 100 mM aniline

- Urea buffer: 8 M urea in 0.4 M NH4HCO3

- 10 mM TCEP

- 12 mM iodoacetamide

- 1.5 M sodium chloride

- 80% acetonitrile

- 50% acetonitrile

- 25 mM NH4HCO3

- 100 mM NH4HCO3

# Procedure

- Prepare hydrazide pipette tips

  - Push one frit into a disposable automation research tip (DART)

  - Load 200 µL hydrazide resin (50% slurry) into each pipette tip

  - Force any residual liquid out of the tip

  - Push one 5 mm diameter frit into the tip to secure the hydrazide resin between the two frits

  - Wash each tip 5x with 200 µL deionized water

  - Condition each tip 5x with coupling buffer

  - Note: Discard tips with slow flow rates due to high resistance

- Couple glycoproteins to hydrazide resin

  - Note: The loading capacity of hydrazide beads is ~ 40 µL serum/200 µL hydrazide beads (50% slurry), and the average glycopeptide yield per 40 µL of serum is 20 µg.

  - Dilute serum 1:1 with oxidation buffer

  - Oxidize with oxidation buffer containing 100 mM aniline for 1 h at room temperature in the dark

  - Buffer exchange with coupling buffer using Zeba spin desalting column

  - Slowly aspirate serum sample into hydrazide tips and dispense into 96-well plate for 30 min using Versette

- Denature, reduce, alkylate and proteolyze glycoproteins

  - Wash serum coupled to the hydrazide tips via their oxidized glycans with 3 mL of urea buffer

  - Reduce disulfide bonds with 10 mM TCEP for 30 min

  - Alkylate with 12 mM iodoacetamide for 15 min in the dark at room temperature

  - Wash with 3 mL of urea buffer

  - Digest with trypsin (1:120) in 100 mM NH4HCO3 for 1 h

- Remove residual non-glycopeptides released by trypsin digestion

  - Wash hydrazide tips (containing conjugated glycopeptides) with 6 mL of 1.5 M sodium chloride

  - Wash hydrazide tips with 6 mL of 80% ACN

  - Wash hydrazide tips with 6 mL of deionized water

  - Wash hydrazide tips with 6 mL of 25 mM NH4HCO3

- Release N-linked glycopeptides

  - Release glycopeptides with 1500 U PNGase F in 25 mM NH4HCO3 for 1 h at room temperature

  - Wash tips 3x with 50% ACN

  - Combine eluents and dry in a vacuum centrifuge

- Reconstitute samples

  - Re-constitute in 1% TFA

  - Measure peptide concentration using NanoDrop

  - Dilute to working concentration with 2% ACN/0.2% formic acid

# Referenced Documents

Chen J, Shah P, Zhang H. Solid phase extraction of N-linked glycopeptides using hydrazide tip. Analytical Chemistry. 2013 (85):10670-10674. PMID: 24079330